| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | DIFFERENTIAL SCANNING CALORIMETER | Application example: Measurement of PET granulate Technical specification Temperature range: -150 up to 600°C Heating and cooling rates:0,001 up to 1000 K/min Temperature accuracy: +/- 0,2 K Temperature precision: +/- 0,02K Digital resolution:16.8 million points Resolution:0,03 µW atmospheres: inert, oxidizing (static, dynamic) Measuring range:+/-2,5 up to +/-250 mW Calibration materials: included Calibration: recommended 6-month interval Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |