| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | X-RAY DIFFRACTION (XRD) | Application Thin Film Analysis Materials Research Screening and Large Area Mapping Features - Millimeter sized beam with high brilliance and ultra-low background - Green design with low power consumption, no water consumption and extended life components - Mapping of samples with a weight of up to 5 kg - Large area mapping of samples with up to 300 mm size. Technical specification TWIST-TUBE Easy switch between point and line focus Available anodes: Cr, Cu, Mo, Ag Max. Power and filament: up to 3 kW depending on anode material (0,4 x 16 mm² ) IµS Microfocus Source Power load: up to 50 W, single-phase power Beam sizes down to 180 x 180 µm². Maximum integrated flux 8 x 10⁸ cps at mirror exit. Beam divergence down to 0,5 mrad Turbo X-Ray Source (TXS) Line focus, 0.3x3 mm² Focal brightness of 6 kW/mm² Anode materials: Cu, Co, Cr, Mo Max. voltage 50 kV, max. power depending on anode material: Cr 3.2 kW, Cu/Mo 5.4 kW, Co 2,8 kW Pre-Aligned Tungsten filament UMC Stages Family of sample stages x,y for sample translation of up to +/- 150 mm z-Drive with travel of up to 50 mm Phi drive with infinite rotation Max. Psi inclination up to 55° Max. weight (at center position) : 50 kg Centric Eulerian Cradle (CEC) Five degrees of freedom sample stage: x,y for sample translation of +/-40 mm z-Drive for height alignment Phi drive with 360° rotation Psi drive and angular range from -11° to 98° Max. weight load: 1 kg Various stage attachment available. Non-ambient Temperature: Ranging from ~12 K up to ~2500 K Pressure: 10-⁴ mbar up to 100 bar Humidity: 5% to 95% RH Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | pcs | 1 |