| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | X-RAY FLUORESCENCE ANALYZER | Features Speedy, Small, Simple, Smart, Safe Application Non-destructive Thickness and Composition Analysis of Coatings on Steel Fast, easy and safe control of Zn-Ni galvanization baths Solid Waste Combustors and Incinerators in Solid Waste Management and Remediation Services Technical specification Basic Items Principle: Energy dispersive X-ray fluorescence spectrometry Target application: RoHS, ELV, Halogen Free Meas. Elements: 13Al - 92U Sample type: Solid, Liquid, Powder X-ray generator X-ray tube: Max 50kV, 0.2mA X-ray irradiation size: 1.2mm, 3mm, 7mm (Automatic switching) X-ray primary filter: 4 types (Automatic switching) Detector Type: SDD (Silicon Drift Detector) Signal processor: Digital pulse processor Sample chamber Atmosphere: Air Sample observation: CCD camera Utility Operation: PC Power supply: AC adapter (100-240V, 50/60Hz) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |