| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | X-RAY FLUORESCENCE ANALYZER | Features Speedy, Small, Simple, Smart, Safe Application Non-destructive Thickness and Composition Analysis of Coatings on Steel Solid Waste Combustors and Incinerators in Solid Waste Management and Remediation Services analysis for the Electronics Industry Technical specification Basic Items Principle: Energy dispersive X-ray fluorescence spectrometry Target application: RoHS, ELV, Halogen Free Meas. Elements: 13Al - 92U Sample type: Solid, Liquid, Powder X-ray generator X-ray tube: Max 50kV, 0.2mA X-ray irradiation size: 1.2mm, 3mm, 7mm (Automatic switching) X-ray primary filter: 4 types (Automatic switching) Detector Type: SDD (Silicon Drift Detector) Signal processor: Digital pulse processor Sample chamber Atmosphere: Air Sample observation: CCD camera Chamber size: 460 x 360 x 150 mm [W x D x H] Utility Operation: PC Power supply: 100-240V, 50/60Hz Dimensions: 590 x 590 x 400 mm [W x D x H] Weight: 60 kg Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |