| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CARBON / WATER ANALYZER | APPLICATION EXAMPLES cement, gypsum, limestone, minerals, ores, soil, ... Technical specification Measured elements: carbon dioxide, water Furnace alignment: horizontal Sample carrier: quartz boats Field of application: construction materials, environment / recycling, geology / mining Furnace: resistance furnace with quartz tube, adjustable up to 1000 °C Process of measurement: fixed temperature, fixed carrier gas, no changing during measurement Detection method: solid state infrared absorption Number of IR cells: 1 - 2 Material of IR path: gold Typical analysis time: 2 - 3 min Chemicals required: magnesium perchlorate, sodium hydroxide Gas required: nitrogen 99.995 % pure (2 - 4 bar / 30 - 60 psi) Power requirements: 230 V, 50/60 Hz, max. 10 A, 2300 W Dimensions (W x H x D) 55 x 80 x 60 cm Weight: ~ 65 kg Required equipment: PC, monitor, balance (resolution 0.0001g) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |