| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | AMBIENT HYDROGEN SULFIDE MONITOR | Description Ambient H2S monitor measures SO2 converted H2S through oxidation catalyst based on Ultra Violet Fluorescence. In order to reduce measurement error by water concentration fluctuation in ambient, humidifier is set at sample line. It enables long-term stability according to stabilizing catalytic reaction. Features Monitor H2S with high sensitivity range in ambient air continuously. Set humidifier at sample line to stabilize catalytic reaction. Technical specification Measurement component: Hydrogen sulfide (H2S) in ambient air Measurement principle: Oxidation catalyst + Ultra Violet Fluorescence Measurement range: 0-0.1/0.2/0.5/1.0 ppm Lower detectable limit: 2 ppb Repeatability: ±3.0% of F.S. Linearity: ±2.0% of F.S. Zero drift: ±2% of F.S./day Span drift: ±5% of F.S./day Response time (T90): 180 sec. or less (from CU-1converter unit) Sample gas flow rate: approx. 0.7L/min Indication: Measuring value, alarm Inpt/Output: 0-1V/0-10V/4-20mA (2 systems: either (1) momentary value and integrated of (2) moving average value) Contact input/output RS-232C (option) Temperature/Humidity 0-40°C / 85% or less Power: 100/110/115/120/220/230/240 VAC, 50/60Hz Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |