| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CONTINUOUS PARTICULATE MONITOR WITH X-RAY FLUORESCENCE | Description Continuous elemental and particulate mass analysis with automatic sampling Application Air Quality Monitoring Brake Dust Measurement Features - Continuous analysis of PM mass and the elemental concentration by a single unit directly in the field. - Adoption of world proven technologies: X-ray fluorescence & Beta-ray attenuation. - Installed CMOS camera enables observation of collected particulate sample on the filter. - User friendly display and operations Technical specification Measured object: Particulate matter (PM10, PM2.5, TSP) Measurement content: Particulate mass concentration and element concentration Flow rate: 16.7L/min Sampling pump: Linear drive system, externally installed Filter tape: None-woven PTFE fabric filter Spot tape interval: 20/25/50/100mm selectable Filter tape replacement interval Approx. 1 month (In case of 100mm spot interval) Ambient operation temperature: 10˚C~30˚C Relative humidity: 0~80% RH noncondensing Altitude: 1000m or less Power supply: AC100V~240V ±10%, 50/60Hz±1% Power consumption: Approx. 400VA External dimension: 430mm(W)×550mm(D)×285mm(H) (without sampling pipe and measurement head) Weight: Approx. 40kg (Without sampling pipe and measurement head) Data output: CSV file (Average PM mass and elemental concentration) External connection: EthernetTM, USB, RS-232C* (option) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |