| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | PLANETARY BALL MILL | Applications pulverizing, mixing, homogenizing, colloidal milling, mechanical alloying, mechano synthesis, nano grinding Field of application agriculture, biology, chemistry, construction materials, engineering / electronics, environment / recycling, geology / metallurgy, glass / ceramics, medicine / pharmaceuticals Technical specifications Feed material: soft, hard, brittle, fibrous - dry or wet Size reduction principle impact, friction Material feed size: < 10 mm Final fineness: < 1 µm, for colloidal grinding < 0.1 µm Batch size / feed quantity: max. 1 x 220 ml, max. 2 x 20 ml with stacked grinding jars Speed ratio: 1:-2 Sun wheel speed: 100 - 650 min-1 Effective sun wheel diameter: 141 mm Type of grinding jars: “comfort", optional aeration covers, safety closure devices Material of grinding tools: hardened steel, stainless steel, tungsten carbide, agate, sintered aluminum oxide, silicon nitride, zirconium oxide Grinding jar sizes: 12 ml / 25 ml / 50 ml / 80 ml / 125 ml / 250 ml / 500 ml Setting of grinding time digital, 00:00:01 to 99:59:59 Interval time: 00:00:01 to 99:59:59 Pause time: 00:00:01 to 99:59:59 Storable SOPs: 10 Interface: RS 232 / RS 485 Drive: 3-phase asynchronous motor with frequency converter Drive power: 750 W Electrical supply data: different voltages Power connection: 1-phase Power consumption: ~ 1250W (VA) W x H x D closed: 640 x 480 (780) x 420 mm Net weight: ~ 86 kg Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |