| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | PULSED LASER AND LED LIGHT SOURCES | Features - The widest range of interchangeable heads - Short optical pulses from <50ps optical FWHM - Repetition rated up to 100MHz - Wavelengths from 250nm to NIR - No additional optimization sources required in normal use - Diode operating features full accessible for advanced applications Technical specification Repetition rates:10kHz,20kHz,50kHz,100kHz,250kHz,500kHz,1MHz,2MHz,4MHz, 8MHz,10MHz,16MHz,20MHz,25MHz, 50MHz,80MHz,100MHz or trigger input (single -shot to 50MHz) subject to attached head Trigger input: pulse amplitude+0.5V to +5V trigger threshold software programmable from+0.2V to 2V Interlock:2-pin connector. contacts must be short-circulated to enable emission Connection to head: 1.5m cable User Interface: LCD display (stand-alone operation) or software (PC control) PC interface: USB 2.0 with integral hub for downstream connection to other USB peripherals Power requirement:90V to 260V AC ,50/60Hz, 100VA Operating temperature: +15°C to +30°C (ambient) Weight &dimension :3.1kg 234*255*92mm Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |