| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | FIBER OPTIC TYPE CHEMICAL CONCENTRATION MONITOR | Features - Reduced Sizes for More Flexible Layout - High-stability, In-line Measurement of High Temperature Chemical Solutions (20 to 80℃). - Significantly Reduced Background Correction Frequency. - A Single Monitor is Capable of Measuring Up to Six Types of Chemical Solutions (solution types or ranges). - Capable of Outputting Measured Values of Up to Six Components (Serial Output). Technical specification Measurement principle: Absorption spectroscopic method Calculation principle: Temperature compensation type multivariate analysis Measurement target: NH3/H2O2/H2O (representative specification) Measurement range: NH3：0.00～1.20mass%、H2O2：0.00～5.50mass% H2O：93.3～100.0mass% Repeatability: NH3：±0.05mass%、H2O2：±10mass%、H2O：±1.0mass% Chemical temperature: Within ±1℃/1 hour Sample solution temperature: 20〜80℃ Measurement cycle: Approx. 3 seconds Air (for operation and purge): Connection port: 4 mm O.D. quick joint, Pressure: 0.2 MPa ±0.02 Mpa Power source: 100～230V AC（Single-Phase）、50/60Hz Power consumption: Approx. 85 VA (transient electric current at the time of the start is excluded) Weight:(Monitor) Approx. 3.6 kg (Lamp unit) Approx. 2.8 kg (Sample cell) Approx. 1.2 kg Ambient temperature: (Monitor, Lamp unit) 20°C to 30°C (Sample cell, Optical fiber) 20°C to 35°C (Sudden temperature change should be avoided, within ±1°C/1 hour) Ambient humidity: (Monitor, Lamp unit, Sample cell) 40% to 70% Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |