| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | FIELD-INSTALLATION TYPE ELECTRIC CONDUCTIVITY METER (FOUR-WIRE TRANSMISSION, FOR HIGH-CONCENTRATION) | Features Tough - Responds to harsh conditions at the site process Operating temperature range(Transmitter)：-20 - 55℃ Rated power supply voltage: AC 90 - 264 V 50/60 Hz Robust aluminum die cast (Protection code：IP65) - CE Marking and FCC Compliance suitable for use at overseas sites Intelligence - Convenient function to display liquid temperature at the same time（Transmission output available） - Sensor error can be detected immediately by self-diagnosis function - Designed to be environmentally friendly (Pb-free) - Sea water salinity and NaCl concentration conversion function - Automatic HNO3/H3PO4/NaOH concentration conversion function Easy maintenance - Easy to view big display - Easy operation - Calibration log memory - Screw terminals that do not fall off are used in the terminal block for wiring Technical specification Temperature: 0℃ to 100℃ (display range: −30℃ to 160℃) Sensors in combination: 4-electrode method conductivity sensor of cell constant 0.1/cm, or 1.0/cm Salinity conversion capability Seawater: 0.00% to 4.00% NaCl: 0.0% to 20.0% Concentration Conversion capability NaOH: 0.00% to 5.00% HNO3: 0.00% to 5.00% H3PO4: 0.00% to 5.00% Option 1 to 4: 0.00% to 100.00% Display resolution Temperature: 0.01℃ Performance Temperature Repeatability: ±0.1℃ (response for equivalent input) Linearity: ±0.5℃ (response for equivalent input) Operating temperature range: −20℃ to 55℃ (without freeze) Operating humidity range: Relative humidity: 5℃ to 90℃ (without condensation) Storage temperature: −25℃ to 65℃ Power supply: Rated power supply voltage: 100 V to 240 V AC ±10% 50/60 Hz Power consumption: 15 VA (max) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |