| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Multi-Parameter Benchtop Meter | Description Easy-to-Use and Accurate Multi-Parameter Benchtop Meter Features Capable of measuring up to 7 parameters including pH, ORP, conductivity, TDS, salinity, resistivity and temperature A 1000-item memory for measurement and calibration allows for efficient data documentation. Records are kept with date/time for traceability. Applications pH, Oxidation-Reduction Potential (ORP), Conductivity, Total Dissolved Solids (TDS), Salinity and Resistivity with Temperature Measurements Technical specification Measurement Range -2.00 – 20.00 pH;-5 °C – 110 °C;-2000.00 – +2000.00 mV Measurement Resolution 0.1 mV;0.1 °C;0.01 pH Accuracy ±0.3 °C;±0.5 mV;±0.01 pH Display: 6.5" Segment & Dot matrix LCD with backlight Operation: Capacitive touch, ABS top housing, standalone electrode holder Communication: RS232, USB Device (included) Construction: AC adapter (included) Design Features: i-Steward, Two Independent Channels, Calibration due alarm, 1,000 measurement memory Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |