| Item No. | Item Name |  | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | EC BENCH METER | Description Highly Reliable Conductivity/TDS/Salinity/Resistivity Benchtop Meter for Standard Laboratory Applications Applications Conductivity, Total Dissolved Solids (TDS), Salinity and Resistivity with Temperature Measurements Features - Selectable reading reference temperatures of 20 or 25 ºC with linear curve options and automatic temperature compensation ensures accurate results. - A 1000-item memory for conductivity measurements and calibration trail allows for efficient data documentation. Standard RS232 and USB interface allows connection to external devices. Technical specification Accuracy: ± 0.5 % Reading ± 2 LSD Display: 6.5" Segment & Dot matrix LCD with backlight Operation: Capacitive touch, ABS top housing, standalone electrode holder Communication: RS232, USB Device (included) Construction: AC adapter (included) Design Features: i-Steward, Calibration due alarm, 1,000 measurement memory Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document |  | pcs | 1 |