| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Dissolved Oxygen (DO) Meter | Description High-Performance, Low-Maintenance Dissolved Oxygen (DO) Meter with Optical Technology for Accurate Measurements Application Field and lab Dissolved Oxygen measurement Features - Large backlit LCD screen provides easy to read results - even in low light environments. Ideal for secure field use with a meter stand, wrist strap and durable IP54 housing. - Optical DO probe requires minimal maintenance and offers immediate measurement readings. No warm up or sample preparation needed. - The 99-measurement library stores data for future reference and allows for easy one-touch recall of the last calibration data. Technical specification Measurement Range: 0.00 – 20.0 mg/L(ppm);0 – 50°C Accuracy: ±0.2 mg/L(<8 mg/L) or ±0.3 mg/L(8~20 mg/L);±0.3 °C Measurement Resolution: 0.01 mg/L;0.1%;0.1 °C Display: Backlit Color Liquid Crystal Display (LCD) Operation: 4 AAA batteries (included) Construction: ABS Housing Design Features: 99 measurement memory, last calibration data recall, auto barometric pressure measurement, auto temperature compensation with manual salinity compensation Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |