| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Portable Dissolved Oxygen (DO) Meter | Description Convenient Portable Dissolved Oxygen (DO) Meter Applications Dissolved oxygen (DO) measurement Features - Easy-to-use and accurate with a simple calibration process, automatic temperature compensation and fast results. - The galvanic electrode can be used immediately after being powered on without the typical wait time associated with dissolved oxygen meters. - The 30-measurement library stores data for future reference and allows the user to easily recall the last calibration data with one quick touch. Technical specification Measurement Range: 0.00 – 19.99;20.0 – 45.0 mg/L;200 – 400%;0 – 50 °C Accuracy: ± 0.3 mg/L;± 1%;±0.3 °C Measurement Resolution: 0.1 mg/L;1 mg/L;0.1%;1%;0.1 °C;0.1 ppm;1 ppm Display: Liquid crystal display (LCD) Operation: 4 AAA batteries (included) Construction: ABS housing, wrist strap Design Features: 30 measurement memory Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |