| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Multi-parameter Water Quality Checker | Features Control Unit Design for Field Operations - Auto-calibration feature provides hassle free calibration of pH, dissolved oxygen, conductivity, turbidity and depth. - Shock resistant cover designed for rough treatment in the field and is easily cleaned. - Cable can be easily connected and disconnected with quick-connect fitting. #### Data Management - Auto hold function freezes average data values on the screen to offer more time to verify or transcribe data. - Diagnostic functions notify the user of errors. - Integral USB connection for data transfer to a PC. USA cable is sold separately and includes software. - Selectable measurement units allow the operator to report data without the need to convert data to desired units of measure. - Multiple sensors allow for the measurement of 11 parameters simultaneously (pH, pH(mv), ORP, DO, COND, Salinity, TDS, Seawater Specific Gravity, Temperature, Turbidity, Water Depth). Technical specification Sensor probe Measurement Temperature: -10 to 55C Maximum sensor diameter: Approx.96mm Probe length:Approx.340mm Measurement depth:Max.30m Control unit Outer dimension:115(w)*66(D)*283(H)mm Mass: Approx 800g Storage temperature:-10 to 60 C Ambient temperature:-5 to 45C PH Range:0 to 14 Resolution:0.01PH Repeatability:0.05PH Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |