| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Benchtop PH meter | Description High Performance Benchtop pH Meter Ideal for Accurate Everyday Measurements in the Laboratory Applications pH, oxidation-reduction potential (ORP) measurement Features A user-friendly bench meter with an innovative design, including an adjustable standalone electrode holder and large touch-screen, backlit LCD display. Technical specification Measurement Range -2.00 – 20.00 pH;-30 – 130°C;-2000.00 – +2000.00 mV Measurement Resolution 0.1 °C;0.001 pH;0.01 mV Accuracy ± 0.03% mV;± 0.1 °C;± 0.002 pH Display: Backlit liquid crystal display (LCD), touchscreen Operation: AC adapter (included) Communication: RS232 and USB (included). GLP/GMP data output with real-time clock. Construction: ABS housing, standalone electrode holder, replaceable in-use cover Design Features: 3 endpoint modes, continuous measurement mode, up to 9 point calibration, 1,000 measurement memory Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | pcs | 1 |