| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Benchtop PH meter | Description An Advanced, Research-Grade Benchtop pH Meter Offering Accurate, Repeatable Results Application pH, oxidation-reduction potential (ORP) with Temperature Measurements Features A 1000-item memory and password management for GLP documentation. Provides connectivity and data output capabilities with RS232 and USB interface. Equipped with a 6.5-inch large LCD display and a touch keypad, offers a simple and precise control experience Technical specification Measurement Range -2.000 – 20.000 pH;-10.0 °C – 125.0 °C;-2000.00 – +2000.00 mV Measurement Resolution 0.1 mV;0.1 °C;0.001 pH Accuracy ±0.3 mV;± 0.002 pH;±0.3 °C Display: 6.5" Segment & Dot matrix LCD with backlight Operation: Capacitive touch, ABS top housing, standalone electrode holder Communication: RS232, USB Device (included), GLP/GMP data output with real-time clock Construction: AC adapter (included) Design Features: i-Steward, 3 endpoint modes, calibration due alarm, 1,000 measurement memory, ten most recent calibrations, password management Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | pcs | 1 |