| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | PORTABLE PH METER | Description Durable, Waterproof Battery-Operated pH Meters Ideal for Field Testing Application pH, Oxidation-Reduction Potential (ORP) measurement Features Rechargeable lithium battery provides 40 hours of uninterrupted use and more than 300 charge cycles - eliminating the need to change out batteries often. Technical specification Measurement Range: -5 °C – 110 °C;-2.00 – 16.00 pH;-1999 – 1999 mV Accuracy: ±0.5 °C;±1 mV;±0.01 pH Measurement Resolution: 0.1 °C;0.01 pH;1 mV Display: Backlit Liquid Crystal Display (LCD) Operation: Rechargeable lithium battery Communication: Built-in micro-USB port Construction: IP67 waterproof plastic housing, IP67 probes, rubber cover Design Features: Data storage of up to 1000 items Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |