| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | ELECTRON MICROSCOPE ANALYZERS(Electron Backscatter Diffraction Analysis) | Application Advanced Phase Identification for mineralogical samples Microstructure characterization of a microprocessor wire bonding Nickel-based superalloy microstructure characterization Features - offers the best spatial resolution down to 1.5 nm - operates with low probe currents without compromising speed and/or data quality - the only TKD solution working in Ultra-High Resolution mode of SEMs using immersion lens technology Technical specification High pixel resolution High angular resolution for HR-EBSD applications High quality optics Kikuchi patterns with low distortions and aberrations Low functioning temperature / low CCD dark current Low noise patterns with better indexing quality High efficiency / quality phosphor screens Produce high quality/definition Kikuchi patterns High precision guiding system Motorized screen positioning with precision better than 10 µm – no detector/ screen rotation Collision sensor Automatic retraction of detector at a speed of 10mm/s (audio alarm present) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |