| Item No | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | OXYGEN/NITROGEN/HYDROGEN ANALYZERS | Features Equipped with various navigation functions Rich lineup of options Fast measurement Applications Oxygen and Nitrogen determination in Boron Nitride Oxygen and Nitrogen determination in Aluminum Nitride The Hydrogen Analysis of Solid Samples Hydrogen Analysis in steel and metals: Solid Extraction or Fusion Sample Preparation and Sampling Methods for the Gas Analysis of Steel Quality Assurance for the Analysis of Steel by Gas Component Analysis Analytical Chemistry in Chemicals Battery Evaluation in Electrical Equipment Law Nitrogen Determination in Steel Technical specifications Measurement target element: oxygen/Nitrogen/Hydrogen Measurement range: Oxygen: Low : 0.1ppm-0.3% or High : 0.1%-5% Nitrogen: Low : 0.1ppm-0.3% or High : 0.1%-3% Hydrogen: 0.08ppm-0.25% Sample weight:1g Cycle time: Approx. 160 sec Required gas He/Ar : Purity greater than 99.995%, Pressure 0.35 MPa (Stainless steel tube (O.D. 3 mm) and suitable connector fitting within 3 m from unit Dry air or N2 as operating gas: Pressure 0.45 MPa (Nylon pipe (O.D. 6 mm) and suitable connector fitting within 5 m from unit) Dimensions 655 x 710 x 725 mm [W x H x D] (Sample window is positioned at 650 mm from table) Cleaner : 420 × 778 ×500 mm[W x H x D] / Radiator unit : 526 × 556 × 360 mm[W x H x D] Weight 197 kg (Furnace unit : 127 kg and Measurement unit : 70 kg) Cleaner : 28 kg / Radiator unit : 33 kg Power Main unit : AC 200/220/230 ±10% 50/60 ±1 Hz 12 kVA (MAX) Vacuum cleaner : AC 100 V (Step-down transformer included) 50/60 ±1 Hz 1.5 kVA (MAX) Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |