| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CS/ONH-ANALYZER | Description Clean, Fast & Efficient Combustion Application Iron, Steel, Cobalt & Nickel, Cast Iron, Case Hardening, Metal Processing ,Cement, Limestone, Lime and Dolomite, Ores & Minerals, Glass & Ceramics Technical specification Sample Types: Inorganic, dry, any form or shape Typical Samples: Ferrous and nonferrous metals and their alloys, ores, cement, limestone, glass, ceramics, soils Sample carrier: Ceramic crucibles (standard form) Sample weight: 0.1 – 1 g (typical 0.5 g) Furnace: High frequency induction furnace, power adjustable Carrier Gas: Oxygen 99.95% purity (99.995% required for trace carbon) 3.5 bar (50 psi) min. pressure Compressed air: 5 bar (72.5 psi) min. pressure, oil-, water- and particle-free Speed: Analysis Time 40 s (nominal) Complete Cycle Time: < 1 min (typical) Noise Emission< 50 db Calibration: Single and multipoint calibration with solid standards Reagents: Magnesium perchlorate, Sodium hydroxide on support, Platinized silica Power Supply 230 VAC (± 10 %); L1 N; 50-60 Hz; 16 A; 2.7 kVA 208 VAC (± 10 %); L1 L2; 50-60 Hz; 16 A; 2.7 kVA Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |