| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CS/ONH-ANALYSIS (CS/ONH | Description High-end analyzer, designed for automatic and precise determination of O/N/H Applications Iron, Steel, Cobalt, Nickel, Ferroalloys, Copper Production, Titanium, Zirconium and other Refractory Metals, Metal Processing, Aluminum and Magnesium, Magnetic Materials, Powder Metallurgy and Additive Manufacturing, Glass & Ceramics, Welding and allied Processes Technical specification Detectors Oxygen :Nondispersive IR absorption of CO, no moving parts, dual-range detectors with reference beam and on-board linearization Nitrogen and Hydrogen: Thermal conductivity detector with reference channel and adjustable gain amplifier Argon and Hydrogen: Mass spectrometer 1-100 amu, single quadrupole, optimized EI source and channeltron detector Carrier Gases: He (O, N), Ar (O, N), N2 (H) each 99.995 % purity (99.9990 % for trace analysis) Calibration Gas: Pure gases or certified mixtures (99.999 % for each compound), 10 individual volumes in the gas dose device to cover a huge range Sample Dimensions: Electrode Furnace: 8 x 8 mm cross section (pieces, chips, pins, granules), sample ports available for special applications Cooling Water: Electrode Furnace: 4 l/min at 3 bar supply pressure IR Furnace: 1 l/min at 3 bar supply pressure, 15 – 20 °C inlet temperature, 100 – 240 V / 50 – 60 Hz Power supply: 400 – 480 VAC (± 10 %), 50 – 60 Hz, 32 A, 7 kVA, 1 Phase plus N or 2 Phase, grounding wire Dimensions and weight: 80 x 60 x 83 cm (W x D x H), ~130 kg*) Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |