| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CS/ONH-ANALYSER | Description Avoid embrittlement, hydrogen-induced cracking and other costly failures Accurate and fast diffusible Hydrogen measurements Application INDUSTRIAL, METALS, WELDING INDUSTRIAL, AUTOMOTIVE & AEROSPACE MATERIAL RESEARCH, ALLOY DEVELOPMENT Technical specification Detector Thermal conductivity detector with reference channel and adjustable gain amplifier Furnaces Infrared heated: IR furnace up to 900 ºC, i.d. of quartz tube 30 mm, water cooled, optional thermocouple kit for direct sample temperature readings Resistance Heated (option): Additional resistance heated furnace up to 1100 ºC, i.d. of quartz tube 18 mm Carrier Gas: Nitrogen 99.995% purity, min. 2 bar (~50 psi), 99.9990% purity for trace analysis Calibration Gas: With pure gases (H2 or He) or certified mixtures (each component 99.999% purity), automatic gas dose calibration system with 10 individual volumes Cooling Water: 1 l/min @ 3 bar (44 psi) Power supply: 230 VAC (± 10 %), 50-60 Hz, 2200 VA Dimension and weight: 630 x 700 x 670 mm (W x D x H), weight ~ 50 kg Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |