| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | LASER SCATTERING PARTICLE SIZE DISTRIBUTION ANALYZER | Application Maximizing Soy Milk Quality Attributes through Particle Size Analysis Particle Analysis of Mayonnaise Particle Size Analysis of Piezoelectric Ceramics Laser Diffraction Analysis of Lithium Ion Battery Material Features Advanced Detector Design Superior Instrument-to-Instrument Precision Automatic Laser Alignment in Seconds Guaranteed Accuracy and Precision Technical specification Measurement Principle: Mie scattering and Fraunhofer diffraction Measurement range: 10 nm - 5000 μm Measurement Time: Typical measurement takes 60 seconds from liquid filling, sampling and measurement to rinsing. Measurement Method: Circulation measurement or fraction cell measurement (Fraction cell is optional) Sample Quantity: Approximately 10 mg - 5 g (Depending on the particle size, distribution and density) Dispersing Volume: Approximately 180 mL for standard pumping system, 5/10/15 mL for FractionCell accessory, Manual filling : 35 mL, Automatic filling : 40 mL for MiniFlow accessory. Approximately 1 L of LiterFlow option. Communication: USB 2.0 Light Sources: Red solid state 5 mW laser diode (650 nm), Blue solid state 3 mW LED (405 nm) Dispersion System: In-line ultrasonic probe: 30 W, 20 kHz, adjustable levels Circulation pump: Fully automated ﬁll and circulation pumps, 15 adjustable speeds, 4 selectable fill levels, 15 selectable circulation speeds (max: 10 L/min) Operating Conditions: 15〜35°C (59 to 95°F), relative humidity 85% or less (no condensation) Power: AC 100-240V 50/60Hz, 300VA Dimensions: 705 (W) × 565 (D) × 500 (H) mm Mass: 54 kg Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |