| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | LASER SCATTERING PARTICLE SIZE DISTRIBUTION ANALYZER | Application Maximizing Soy Milk Quality Attributes through Particle Size Analysis Milk Homogenization Evaluation by Particle Analysis Particle Characterization of Powdered Sugar Particle Analysis for Power Plant Emission Control Particle Analysis of Road Materials Features - Range: 0.1 - 1,000 µm - Compact Size - Fast Analysis - Powerful Software - Ease of Use Technical specification Measurement Principle: Laser diffraction and Mie light scattering theory Size Measurement Range: 0.1-1,000 μm Analysis Time: Typical measurement takes about 10 seconds from "Measure" to "display the result". Analysis Materials: Powders, slurries, emulsions etc. Measurement Output: Particle size and size distribution, size related theoretical calculated values Measurement Method Wet method: Liquid dispersed particles with flow sampling system Required Sample Amount: 10 mg-5 g (depending on the sample size, distribution and materials) Wet Flow System Liquid Volume: Approximately 130-230 mL Organic Solvent Compatibility: Available in solvent resistant flow sampling version or fraction cell system (optional) Interface: USB data communication with PC Data Processing/Results Display Desktop or laptop PC/LCD; Printer Operating Conditions: 15° to 35° C 85% RH or less (non-condensing) Power: AC100/120/230V, 50/60Hz, 150VA Dimensions: W 297mm x D 420mm x H 376mm (excluding PC) Mass: Approximately 23 kg Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |