| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | CS/OHN-ANALYSIS (CS/ONH) | Description: Economic Quality & Process Control for O/N/H High-precision, easy-to-use and robust Inert Gas Fusion analyzer Application Iron, Steel, Cobalt, Nickel, Ferroalloys, Copper Production, Titanium, Zirconium and other Refractory Metals ,Metal Processing, Aluminum and Magnesium, Magnetic Materials, Powder Metallurgy and Additive, Manufacturing, Glass & Ceramics Technical specification Sample Types: Inorganic, dry, any form or shape Sample Dimensions: 8 x 8 mm cross section (pieces, chips, pins, granules), different sample ports available for special applications Detectors: Oxygen, Nitrogen and Hydrogen Nondispersive IR absorption of CO, no moving parts, multiple range detectors with reference beam and on-board linearization Thermal conductivity detector with reference channel and adjustable gain amplifier Furnace: Water-cooled Electrode Furnace with Fusion Control Cooling Water: 4 l/min at 3 bar supply pressure Carrier Gas: Ar (O, N), He (O, N), N2 (H) each 99.995 % purity (99.9990 % for trace analysis) Noise Emission: < 55 dB(A) peak Calibration: Single and multipoint calibration, advanced weighting algorithm with error bars and range merging Reagents: Magnesium perchlorate, Sodium hydroxide on support, Copper oxide, rare earth base, Schuetze reagent Power Supply: 400 - 480 VAC (± 10 %), 50-60 Hz, 32 A, 7 kVA, 1 Phase plus N or 2 Phase, grounding wire 230 VAC (± 10 %), 50-60 Hz, 64 A, 7 kVA available as option Dimensions and Weight: 71 x 66 x 78 cm (W x D x H), ~145 kg Documentation - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |