| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | 3D X-RAY MICROSCOPY (XRM) | Application Additive manufacturing Fibers and composites Geology Technical specification X-ray source: 20-160 kV, 16 W max. X-ray detector: 6 Mp active pixel flat-panel, 11 Mp large format cooled CCD, 11 Mp mid format cooled CCD, 8 Mp hi-res cooled CCD Image Formats: Up to 8000 x 8000 x 2300 pixels after a single scan Resolution: 60 nm smallest pixel size, <500 nm low-contrast resolution (10% MTF) Positioning Accuracy: <50 nm for rotation, Anti-vibration granite platform with pneumatic leveling Maximum Object Size: 300 mm in diameter (140 mm scanning size), 400 mm in length, Maximum object weight 20 kg Dimensions: W 1800 mm x D 950 mm x H 1680 mm Weight: 1500 kg Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |