| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | ATOMIC FORCE MICROSCOPE | Features - High-speed imaging with 10 frames/sec with excellent resolution - Revolutionary new workflow-based user interface for ergonomics and ease of operation - New tiling functionality for automated mapping of large sample areas together with the HybridStage - Unique integration with optical microscopy by tip-scanning design and the newly enhanced DirectOverlay 2 mode for most precise correlative microscopy - New Vortis 2 controller with high-speed low-noise DACs and cutting-edge position sensor readout technology - Highest flexibility and upgradeability Technical specifications True atomic resolution on inverted microscope in closed-loop (< 0.015 nm RMS z height noise level) Ultra-low noise level of cantilever deflection detection system < 2 pm RMS free (0.1 Hz - 1 kHz) Highest detector bandwidth of 8 MHz for high speed signal capture Tip-scanning, stand-alone system, with a rigid low-noise design and drift-minimized mechanics The only liquid-safe AFM with integrated vapor barrier, special encapsulated piezo drives and tip-moving design IR deflection detection light source with low coherence Transmission illumination with standard condensers for precise brightfield, DIC and phase contrast Scanner unit: 30 × 30 × 6.5 µm³ scan range with 1.5 µm extra z range with high-speed option Sensor noise level < 0.09 nm RMS in xy 0.04 nm RMS sensor noise level in z Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |